Heritage Statement

136 St Werburghs Road. Chorlton-cum-Hardy. Manchester M21 8UQ

This building is Grade II listed under the Planning (Listed Buildings and Conservation Areas) Act 1990 as amended, for its special architectural or historic interest

The property was listed on 21st October 1989 under English Heritage Legacy ID: 457623

The 2-storey building which was formerly a farm house dates from the late 18th Century and was part of a working farm up to the mid 1980s.

Together with the adjoining outbuildings and various parcels of surrounding land it was known as

Park Brow Farm and under the ownership of the Bailey Family.

The farmhouse, outbuildings, and surrounding land were disposed of in the mid 1980s with the

farmhouse converted into an office and surrounding lands developed for housing development.

The existing barn adjacent to the building is also Grade II listed and was later converted into 3 houses.

Substantial elements of the original fabric of the building (farmhouse) remain in-situ.

These include but are not limited to external walls constructed in an English Garden Wall Bond with other walls constructed in Flemish Bond, gable chimneys, and a number of small pane windows with horizontal sliding sashes.

There are also elements of 20th Century alterations to the building and these include but are not limited to a single-storey addition, replacement bow windows, replacement casement windows.

The listing makes no reference to the treatment of boundaries of the building.

The proposed works involve reconfiguring the previously converted internal footprint to create three self-contained apartments.

The existing staircase and entrance porch would be preserved along with all external window openings.

Proposals do not include external alteration to the fabric of the property and any window replacement would, subject to approval, be on a like for like basis.

The current use of the property is that of a small boutique type of gym – wellness centre and the proposed conversion if successful would not only significantly reduce the current coming and going to the property but transform the property into residential accommodation more in keeping with that in surrounding properties.

Off road parking would be achieved by use of the existing car park to the rear of the premises which would provide 4 car parking spaces.

It is anticipated that occupiers will make use of local public transport links especially the

Saint Werburghs Road Metrolink stop located approx. 300m from the property.

Provision would be made to sympathetically provide bin and cycle storage as indicated on the proposed site plan.