| Contract Name | Storage World | Contract No | 1154 |  |  |
| --- | --- | --- | --- | --- | --- |
| Name of Assessor | David Ford | Signature |  | Date | 13/02/25 |

| Work Activity / Hazard: | Vehicle movement adjacent to existing railway |
| --- | --- |

| Risk Matrix |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Severity |  | Likelihood |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Fatality, Major injury or illness causing long term disability | 5 | Certain or near certain to occur | Severity | 5 | 5 | 10 | 15 | 20 | 25 |  | High | 15 – 25 |
|  |  |  |  |  | 4 | 4 | 8 | 12 | 16 | 20 |  |  |  |
|  |  |  |  |  | 3 | 3 | 6 | 9 | 12 | 15 |  | Medium | 7 – 14 |
| 3 – 4 | Injury or illness causing long term disability | 3 – 4 | Reasonably certain to occur |  | 2 | 2 | 4 | 6 | 8 | 10 |  |  |  |
|  |  |  |  |  | 1 | 1 | 2 | 3 | 4 | 5 |  | Low | 1 – 6 |
| 1 – 2 | Other injury or illness | 1 – 2 | Very seldom or never occurs |  |  | 1 | 2 | 3 | 4 | 5 |  |  |  |
|  |  |  |  |  | Likelihood |  |  |  |  |  |  |  |  |

| Hazards | Persons at Risk | Existing Controls |  | Control Measures |  |  | H / M / L |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Collision with railway structure | Operatives | Currently the existing building is in place. |  | Use of waterfilled barriers. Use of vehicle stop blocks Use of warning markers designating vehicle banksman controlled areas. |  |  | 2 |
| Additional Measures: |  |  |  |  |  |  |  |
| - Ensure all subcontractors using machinery have a thorough risk assessment highlighting the vehicle management plan. - On site trained bankman to be present at all times. - Information regarding control areas to be included within the site induction. |  |  |  |  |  |  |  |
| Method Statement |  |  |  |  |  |  |  |
| Sope of works: Demolition of an existing building and construction of a new 4 story storage unit Responsibilities: - Site Manager - Ensure Compliance with RAMS and supervises mitigation measures. - All Workers - Follow details within the RAMS and ensure any issues are reported to the site manager. Control Measures: Follow the above risk assessment and mark up with control measures. Daily brief board to be utilised and ensure no vehicle movement within the exclusion zone unless a banksman is present. All on site personnel to be briefed of the restrictions within the site induction. |  |  |  |  |  |  |  |
| We The Undersigned Have Received, Read And Understood The Risks And The Specific Precautions Listed Within This Document |  |  |  |  |  |  |  |  |
| Name |  |  | Company |  | Signature | Date |  |  |