Carbon Emissions Reductions Measures Summary

20.12.2024

In summary the following measures are incorporated:

6no. 400 Watt PV Panels per house.

36kW high efficiency gas combi boiler with time and zone temperature control

Decentralised Mechanical Extract Ventilation (dMEV)

Smart Meters

Waste water heat recovery

LED Lighting

High levels of airtightness to achieve no more than 4m3 at 50pa

High levels of insulation with fabric U-Values of 0.12W/m2K on the floor, 0.17W/m2K on the walls, 0.11W/m2K on the roof and 1.3W/m2K on doors and windows

The SAP rating is 93 and band A

The Dwelling Emission Rate is between 8.86 and 8.9 this is lower than Building Regulations New Build Home Target Emission Rate which is 9.46-9.51