To whom it may concern

R:E McCarthy Stone Development, Spath Rd Didsbury

We refer to the above development and in particular the proposed Surface Water site layout GGP Consultants drawing ref: 29359/106 C2.

Civils and Construction Solutions Ltd, were appointed to be the Groundworker for all sub-structure civils works to include all private drainage, foundations and S278 site entrance works for clarity and which was approved by the NHBC Officer and Manchester City council Highways accordingly.

It had been intended to install the surface water Geo-cell storage facility and membrane surround in a soakaway manner such that it would filtrate direct to ground as required. However, during the excavation to required depths we encountered ground water within the excavation and thus made the McCarthy Stone Technical Department aware of the situation and request they review. During this short passage of time, difficulty of continued activity in a confined area it was necessary to make a sump and dewater the excavation of groundwater in this instance.

Under review, it was decided that we connect the proposed surface water Geo-cell storage facility with a Hydro-brake manhole and discharge into the existing combined manhole (which accepted all impermeable areas of the original property and hardstanding) at 5L/s. We were made aware of United Utilities provide a written and confirmed discharge rate of 5L/s into the adjacent public combined sewer via the existing combined manhole within the site.

It was considered that to install the Geo-cell units as a soakaway resting at proposed invert levels yet below the groundwater encountered would have been too high risk in this instance, in light of the aged residents within the development. We trust the above clarifies our position and meets with your approval.

Should you have any queries please do contact me at any time.

Yours

Mr P Barnes

Managing Director