Planning Assessment
Before considering whether the proposal meets the requirements of permitted development defined within Schedule 2, Part 1, Classes A and B of the GPDO, it first has to be determined whether permitted development rights exist for this residential property. The following table clarifies the situation in this particular case.

| Condition(s) withdrawing PD rights? | No |
| --- | --- |
| Article 4 Direction affecting the site | No |
| Conclusion: Are the relevant PD rights intact for this property | Yes |

It therefore now needs to be considered whether the proposal meets the requirements of permitted development contained within Schedule 2, Part 1, Classes A and B of the GPDO.  The following tables set out the relevant requirements/ conditions of the relevant classes of permitted development and whether the proposal complies with them.

Class A – Enlargement, Improvement or Alteration

| GPDO ref: | Requirement Condition | Comment |
| --- | --- | --- |
| A.1 (a) | Has the use as the dwellinghouse been granted only by virtue of Classes M, N, P, PA or Q of Part 3 of Schedule 2? | No |
| (b) | As a result of the works, will the total ground area covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) exceed 50%? | No |
| (c) | Will the height of part of the dwellinghouse enlarged, altered or improved exceed the highest part of the roof of the existing dwellinghouse? | No |
| (d) | Will the height of the eaves of the part of the dwellinghouse altered, enlarged or altered exceed the height of the eaves of the existing dwellinghouse? | No |
| (e) | Will the enlarged part of the dwelling extend beyond a wall which; (i) Forms the principal elevation of the original dwellinghouse? (ii) Fronts a highway and forms a side elevation of the original dwellinghouse? | - No - No |
| (f) | In the case of a single storey extension, would it: - Extend beyond the rear wall of the original dwellinghouse by more than 4 metres in the case of a detached house? - Extend beyond the rear wall of the original dwellinghouse by more than 3 metres in the case of any other dwellinghouse? - Exceed 4 metres in height? | (i) N/A - No – it extends 3 metres (iii) No |
| (g) | For a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, would the enlarged part have a single storey and; - Extend beyond the rear wall of the original dwellinghouse by more than 8 metres in the case of a detached dwellinghouse, or 6 metres in the case of any other dwellinghouse, or - Exceed 4 metres in height? | Not applicable to this application, only applicable to larger home extension applications. |
| (h) | In the case of an addition with more than one storey, would the enlarged part of the dwelling house; - Extend beyond the rear wall of the original dwellinghouse by more than 3 metres? - Be within 7 metres of any boundary of the curtilage of the dwellinghouse being enlarged which is opposite the rear wall of the dwellinghouse? | Not applicable, the proposal is not for an extension over one storey |
| (i) | Where the enlarged part of the dwellinghouse would be within 2 metres of the boundary of the curtilage of the dwellinghouse, would the eaves height of the enlarged part exceed 3 metres? | The eaves are 2.29 metres above ground level |
| (j) | Where the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, would it: (i) Exceed 4 metres in height? (ii) Have more than a single storey? (iii) Have a width greater than half the width of the original dwellinghouse? | Not applicable, the proposal is for a rear extension |
| (ja) | Would the total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraphs ( e) to (j) | The proposed extension will not be joined to an existing extension |
| (k) | Would the development consist of or include; - The construction or provision of a veranda, balcony or raised platform? (i.e. its height would be greater than 0.3 metres above ground level)? - The installation, alteration or replacement of a microwave antenna? - The installation, alteration or replacement of a chimney, flue or soil and vent pipe? - An alteration to any part of the roof of the dwellinghouse? | - No - No - No - Yes – to be assessed under Class B |
| A.2 | Only relating to a dwelling house on Article 2(3) land (Conservation Area, AONB, etc.) | N/A – site not located on Article 2(3) land |
| (a) | Would the works consist of or include the cladding of any part of the exterior of the dwellinghouse with stone, artificial stone, pebble dash, render, timber, plastic or tiles? |  |
| (b) | Would the enlarged part of the dwellinghouse extend beyond a wall forming a side elevation to the original house? |  |
| (c) | Would the enlarged part of the dwellinghouse have more than a single storey and extend beyond the rear wall of the original dwellinghouse? |  |
| (d) | Would the total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraphs (b) and (c) |  |
| A.3 (a) | Would the external materials used in the proposed works (other than those used on a conservatory) be similar in appearance to those used on the exterior of the existing dwellinghouse? | The materials will match the existing as close as possible. |
| (b) | Would any upper floor windows located within a wall or roof slope forming a side elevation of the dwelling house be: (i) Obscured glazed, and non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed | Not applicable |
| (c) | Where the enlarged part of the dwellinghouse has more than a single storey, or forms an upper storey on an existing enlargement of the original dwellinghouse, the roof pitch of the enlarged part must, so far as practicable, be the same as the roof pitch of the original dwellinghouse | Not applicable |

Class B- Additions etc to the Roof

| GDPO ref: | Requirement/ Condition | Comment |
| --- | --- | --- |
| B.1 (a) | Has permission to use the dwellinghouse as a dwellinghouse been granted only by virtue of Class G, M, MA, N, P, PA or Q of Part 3 of this Schedule (changes of use)? | No |
| (b) | Will any part of the dwelling, as a result of the works, exceed the height of the highest part of the existing roof? | No |
| (c) | Will any part of the dwellinghouse, as a result of the works, extend beyond the place of any existing roof slope which forms the principle elevation of the dwellinghouse and fronts a highway | No |
| (d) | Would the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than - (i) 40 cubic metres in the case of a terrace house, or (ii) 50 cubic metres in any other case | - N/A - No |
| (e) | Would the proposal consist of or include – - the construction or provision of a verandah, balcony or raised platform, or - the installation, alteration or replacement of a chimney, flue or soil and vent pipe | - No (ii)No |
| (f)) | Is the dwelling located on article 2(3) land | No |
| (g) | Was the dwelling built under Part 20 of the GPDO? | No |
|  | Has the dwelling been enlarged in reliance ont eh permission granted by Class AA? | No |
| B.2 (a) | Would the external materials used in the proposed works be similar in appearance to those used on the exterior of the existing dwellinghouse? | Yes |
| (b) | Would the enlargement shall be constructed so that – (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension – (aa) the eaves of the original roof are maintained or reinstated; and (bb) the edge of the enlargement closest to the eaves of the original roof shall, so far as practicable, be not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse | (aa) yes (bb) yes - No part extends beyond the outside face of any external wall |
| (c) | Would any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse be – (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed | N/A – no window proposed to be inserted on a wall or roof slope forming a side elevation |

In light of the above, the proposed development would meet the requirements/ conditions of permitted development contained within Schedule 2, Part 1, Classes A and B of the Town and Country Planning (General Permitted Development) Order 2015 (as amended).  All work is within the curtilage of the applicant’s property and there will be no encroachment to the neighbouring property.