Project and Client Details

| Date: | 21.07.22 | ATSM: John Wilkinson |
| --- | --- | --- |
| Bauder reference: | B193311 | Project Address: |
| Client Name: | Mr Glenn Robinson | First Street South |
| Company: | Simpson Haugh | Manchester |
| Tel: |  | M15 6JG |
| Email: | g.robinson@simpsonhaugh.com |  |

Rainfall Details

1) Type of Roof		                                                               Warm (Specify Pitch) / Cold (Specify Pitch)

/ Warm Inverted (<1:80) / Warm Green or Brown (<1:80)

1) Roof Pitch						<1:80  / 1:80 or more (assumed 1:80 if left blank)

2) Building Design Life					      Years (assumed 25 years if left blank)

3) Has the safety factor been set?			Yes / No

If the answer is yes then please state			Safety Factor       (N/A to Roofs with <1:80 slope)

Safety Factor 1.5 - Cat 2 = Normal Buildings

Safety Factor 4.5 - Cat 3 = Higher risk Buildings (Hospital operating theatre – communications centre – buildings where any disruption will be very costly or any other high risk buildings)

Bauder will calculate the drainage required based on the rainfall intensity for the project location.

| Roof Name | Roof Area (m²) | Area of Adjacent surfaces (m²) | Total Area (m²) | Type of Bauder Outlet Required | Diameter of Bauder Outlet Required (mm) | Number of Preferred Outlets |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | 328 |  | 328 | Hotmelt Compact | 100 |  |
| A2 | 222 | 95 | 317 | “ | “ |  |
| A3 | 214 | 97 | 311 | “ | “ |  |
| A4 | 190 | 97 | 287 | “ | “ |  |
| B1 | 255 | 94 | 349 | “ | “ |  |
| B2 | 226 | 94 | 320 | “ | “ |  |
| B3 | 336 |  | 33 | “ | “ |  |
| B4 | 307 | 97 | 404 | “ | “ |  |
| B5 | 249 | 97 | 346 | “ | “ |  |
| D1 | 742 |  | 742 | “ | “ |  |

*See diagram on the following pages on how to calculate effective roof areas.

Note:

If a CLEAR Roof plan showing the direction of falls and the outlet location in either DWG and/or PDF is available then please forward this onto your local Area Technical Sales Manager along with this form.

Flat Roof Catchment Area – L x W	 Pitched Roof Catchment Area – L x (W + 0.5H)

Vertical Wall Catchment Area – 0.5 x L x H        Pitched Roof Valley Catchment Area

– L x (W + 0.5 (H1 – H2))