| COSHH Assessment for: |  |  |  |  |  |  |  |  | Cortex 0778FR Contact Adhesive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ref: |  | 353 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assessor: |  |  | Dominic Reddall |  |  |  |  |  |  | Date: |  |  |  | September 2022 |  |  |  |  |  |  |  | Next Review: |  |  |  |  |  |  |  | September 2025 |  |  |  |  |
| Describe the activity or work process: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| What is the location of the activity or work process? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identify the Persons at Risk: |  |  |  |  |  |  |  |  |  | Employees |  |  |  |  |  |  |  | Contractors |  |  |  |  |  |  |  |  |  |  | Public |  |  |  |  |  |
| Name the substance involved in the process, the quantity and its manufacturer. (A copy of a current safety data sheet for this substance should be attached) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cortex 0778FR Contact Adhesive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Explain why a safer alternative cannot be used: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Substitution is not possible as the substance has been specified by the client |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Classification (tick all that apply): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Physical Hazards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unstable Explosive |  |  |  |  |  | Flammable |  |  |  |  |  |  | Oxidising |  |  |  |  |  |  |  | Compressed Gas |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applicable: |  |  |  |  |  | Applicable: |  |  |  |  |  |  | Applicable: |  |  |  |  |  |  |  | Applicable: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Health Hazards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Environmental |  |  |  |  |
| Toxic |  |  |  |  |  | Corrosive |  |  |  |  |  |  | Irritant |  |  |  |  |  |  |  | Resp. Sens/Health |  |  |  |  |  |  |  |  | Environmental |  |  |  |  |
| Applicable: |  |  |  |  |  | Applicable: |  |  |  |  |  |  | Applicable: |  |  |  |  |  |  |  | Applicable: |  |  |  |  |  |  |  |  | Applicable: |  |  |  |  |
| Hazard Type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gas |  | Vapour |  |  |  |  |  | Mist |  |  |  | Fume |  |  |  |  | Dust |  |  |  |  | Liquid |  |  |  |  |  | Solid |  |  |  | Other: |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Absorption |  |  |
| Route of Exposure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inhalation |  |  |  |  |  | Skin |  |  |  |  |  |  | Eyes |  |  |  |  |  |  |  | Ingestion |  |  |  |  |  |  |  |  | Other (State) |  |  |  |  |
| Explosive Limits and Workplace Exposure Limits (WELs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Long-term exposure level (8hrTWA): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Short-term exposure level (15 mins): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWA 100 ppm 350 mg/m3 2007- 08-01 GB EH40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | STEL 300 ppm 1,060 mg/ m3 Can be absorbed through skin. The assigned substances are those for which there are concerns that dermal absorption will lead to systemic toxicity. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Detail Potential Health Risks: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard statements: H315: Causes skin irritation. H319: Causes serious eye irritation. H335: May cause respiratory irritation. H336: May cause drowsiness or dizziness. H351: Suspected of causing cancer. H373: May cause damage to organs through prolonged or repeated exposure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Control & Recovery Measures Actioned: (for example extraction, ventilation, training, supervision). Include special measures for vulnerable groups, such as disabled people and pregnant workers. Take account of those substances that are produced from activities undertaken by another employer’s employees. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ventilation: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Accidental Release & Spillage Plan : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Do not breathe dust/ fume/ gas/ mist/vapours/ spray. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Prevent spillage into watercourses and sewers. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Special Transport: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Signage, provision of information and instruction: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Brief of this assessment. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fire Precautions/Extinguisher Type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Environmental Considerations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use extinguishing measures that are appropriate to local circumstances and the surrounding environment. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Environmental hazards: Aquatic Chronic 3 - H412 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Storage Required: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Disposal Issues: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| P403 + P233: Store in a well-ventilated place. Keep container tightly closed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | See hazardous waste |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Is health surveillance or monitoring required for foreseeable exposure potential? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Routine health screening |  |  |  |  | No |  |  |  |  | Personal air monitoring |  |  |  |  |  |  | No |  |  |  |  |  |  |  | Background air monitoring |  |  |  |  |  | No |  |  |  |
| Personal Protective Equipment (state type and standard) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item | Tick |  | Requirement |  |  |  |  |  |  |  |  |  |  |  |  |  | Item |  |  | Tick |  |  |  | Requirement |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Chemical splash goggles or face shield. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | If ventilation is inadequate, suitable respiratory protection must be worn. In confined or poorly- ventilated spaces, a supplied-air respirator must be worn. Wear a respirator fitted with the following cartridge: Particulate filter, type P3. ABEK2-P3: Environmental exposure controls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Chemical splash goggles or face shield. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | It is recommended that gloves are made of the following material: Nitrile rubber. It should be noted that liquid may penetrate the gloves. Frequent changes are recommended. For exposure up to 8 hours, wear gloves made of the following material: Viton rubber (fluoro rubber). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Wear suitable protective clothing as protection against splashing or contamination. Wear apron or protective clothing in case of contact. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| First Aid Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Remove affected person from source of contamination. Inhalation: Move affected person to fresh air at once. Ingestion: DO NOT induce vomiting. Get medical attention immediately. Skin contact: Remove contaminated clothing immediately and wash skin with soap and water. Eye contact: Rinse immediately with plenty of water. Remove any contact lenses and open eyelids wide apart. Continue to rinse for at least 15 minutes. Get medical attention if irritation persists after washing. Show this Safety Data Sheet to the medical personnel. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Disposal Instructions for substance and empty containers: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous Waste |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | General Waste |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Return to Depot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Return to Supplier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other/Notes: |  |  |  |  |  |  | Dispose of as hazardous waste in compliance with local and national regulations. Where possible recycling is preferred to disposal or incineration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Is exposure adequately controlled? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |  |  |
| Risk Rating Following Above Controls: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | High |  |  |  |  |  |  | Med |  |  |  |  | Low |  |  |  |