| COSHH Assessment for: |  |  |  |  |  |  |  |  | Rockwool Insulation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ref: |  | 300 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assessor: |  |  | Dominic Reddall |  |  |  |  |  |  | Date: |  |  |  | April 2022 |  |  |  |  |  |  |  | Next Review: |  |  |  |  |  |  |  | April 2025 |  |  |  |  |
| Describe the activity or work process: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rockwool Insulation is used as Thermal Insulation, acoustic insulation and fire protection in building construction applications. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| What is the location of the activity or work process? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| All areas specified by client including cavities, walls, roofs etc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identify the Persons at Risk: |  |  |  |  |  |  |  |  |  | Employees |  |  |  |  |  |  |  | Contractors |  |  |  |  |  |  |  |  |  |  | Public |  |  |  |  |  |
| Name the substance involved in the process, the quantity and its manufacturer. (A copy of a current safety data sheet for this substance should be attached) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Quantities will vary dependant on the roof area & job specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Explain why a safer alternative cannot be used: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The material and quantity has been specified by the client. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Classification (tick all that apply): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Physical Hazards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unstable Explosive |  |  |  |  |  | Flammable |  |  |  |  |  |  | Oxidising |  |  |  |  |  |  |  | Compressed Gas |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applicable: |  |  |  |  |  | Applicable: |  |  |  |  |  |  | Applicable: |  |  |  |  |  |  |  | Applicable: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Health Hazards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Environmental |  |  |  |  |
| Toxic |  |  |  |  |  | Corrosive |  |  |  |  |  |  | Irritant |  |  |  |  |  |  |  | Resp. Sens/Health |  |  |  |  |  |  |  |  | Environmental |  |  |  |  |
| Applicable: |  |  |  |  |  | Applicable: |  |  |  |  |  |  | Applicable: |  |  |  |  |  |  |  | Applicable: |  |  |  |  |  |  |  |  | Applicable: |  |  |  |  |
| Hazard Type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gas |  | Vapour |  |  |  |  |  | Mist |  |  |  | Fume |  |  |  |  | Dust |  |  |  |  | Liquid |  |  |  |  |  | Solid |  |  |  | Other: |  |  |
| Route of Exposure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inhalation |  |  |  |  |  | Skin |  |  |  |  |  |  | Eyes |  |  |  |  |  |  |  | Ingestion |  |  |  |  |  |  |  |  | Other (State) |  |  |  |  |
| Explosive Limits and Workplace Exposure Limits (WELs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Long-term exposure level (8hrTWA): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Short-term exposure level (15 mins): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Workplace exposure limit (WEL) 5mg/m3 gravimetric measure (total inhalable dust) and 2 fibres/ml airborne fibre limit, 8-hour time weighted averages. HSE guidance assumes that the gravimetric measure would be reached before the fibre measure. (Ref. HSE EH40). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Detail Potential Health Risks: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of high speed cutting tools can generate dust. The mechanical effect of coarse fibres in contact with throat, skin or eyes may cause temporary itching/inconvenience. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Control & Recovery Measures Actioned: (for example extraction, ventilation, training, supervision). Include special measures for vulnerable groups, such as disabled people and pregnant workers. Take account of those substances that are produced from activities undertaken by another employer’s employees. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ventilation: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Accidental Release & Spillage Plan : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No Specific measures. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Vacuum cleaner or dampen with water spray prior to sweeping up. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Special Transport: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Signage, provision of information and instruction : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Breifing this assessment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fire Precautions/Extinguisher Type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Environmental Considerations : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Suitable extinguishing media: Water, foam, carbon dioxide (CO2), and dry powder Unsuitable extinguishing media: None |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | None required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Storage Required: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Disposal Issues: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No Specific measures. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rockwool is recyclable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Is health surveillance or monitoring required for foreseeable exposure potential? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Routine health screening |  |  |  |  | No |  |  |  |  | Personal air monitoring |  |  |  |  |  |  | No |  |  |  |  |  |  |  | Background air monitoring |  |  |  |  |  | No |  |  |  |
| Personal Protective Equipment (state type and standard) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item | Tick |  | Requirement |  |  |  |  |  |  |  |  |  |  |  |  |  | Item |  |  | Tick |  |  |  | Requirement |  |  |  |  |  |  |  |  |  |  |
|  |  |  | When working in unventilated areas or during operations which can generate emission of (various) dusts, wearing a disposable face mask in accordance with EN 149 FFP1 is recommended. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Wear goggles if working above shoulders or where there is heavy dust development. Eye protection to EN 166 is advised. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Use gloves conforming with EN 388 to avoid itching. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Cover exposed skin. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| First Aid Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inhalation: Remove from exposure. Rinse the throat and clear dust from airways. Skin: If itching occurs, remove contaminated clothing and wash skin gently with cold water and mild soap. Eye: Rinse abundantly with water for at least 15 minutes. Ingestion: Drink plenty of water if accidentally ingested. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Disposal Instructions for substance and empty containers: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous Waste |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | General Waste |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Return to Depot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Return to Supplier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other/Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Is exposure adequately controlled? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |  |  |
| Risk Rating Following Above Controls: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | High |  |  |  |  |  |  | Med |  |  |  |  | Low |  |  |  |