| COSHH Assessment for: |  |  |  |  |  |  |  |  | Rockwool Acoustic Intumescent Sealant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ref: |  | 301 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assessor: |  |  | Dominic Reddall |  |  |  |  |  |  | Date: |  |  |  | May 2022 |  |  |  |  |  |  |  | Next Review: |  |  |  |  |  |  |  | May 2025 |  |  |  |  |
| Describe the activity or work process: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rockwool Acoustic Intumescent Sealant is used to seal joints and service gaps in compartment walls and penetration seals. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| What is the location of the activity or work process? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| All areas specified by client including cavities, walls, roofs etc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identify the Persons at Risk: |  |  |  |  |  |  |  |  |  | Employees |  |  |  |  |  |  |  | Contractors |  |  |  |  |  |  |  |  |  |  | Public |  |  |  |  |  |
| Name the substance involved in the process, the quantity and its manufacturer. (A copy of a current safety data sheet for this substance should be attached) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Quantities will vary dependant on the roof area & job specification. Manufactured by ROCKWOOL. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Explain why a safer alternative cannot be used: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The material and quantity has been specified by the client. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Classification (tick all that apply): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Physical Hazards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unstable Explosive |  |  |  |  |  | Flammable |  |  |  |  |  |  | Oxidising |  |  |  |  |  |  |  | Compressed Gas |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applicable: |  |  |  |  |  | Applicable: |  |  |  |  |  |  | Applicable: |  |  |  |  |  |  |  | Applicable: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Health Hazards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Environmental |  |  |  |  |
| Toxic |  |  |  |  |  | Corrosive |  |  |  |  |  |  | Irritant |  |  |  |  |  |  |  | Resp. Sens/Health |  |  |  |  |  |  |  |  | Environmental |  |  |  |  |
| Applicable: |  |  |  |  |  | Applicable: |  |  |  |  |  |  | Applicable: |  |  |  |  |  |  |  | Applicable: |  |  |  |  |  |  |  |  | Applicable: |  |  |  |  |
| Hazard Type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gas |  | Vapour |  |  |  |  |  | Mist |  |  |  | Fume |  |  |  |  | Dust |  |  |  |  | Liquid |  |  |  |  |  | Solid |  |  |  | Other: |  |  |
| Route of Exposure: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inhalation |  |  |  |  |  | Skin |  |  |  |  |  |  | Eyes |  |  |  |  |  |  |  | Ingestion |  |  |  |  |  |  |  |  | Other (State) |  |  |  |  |
| Explosive Limits and Workplace Exposure Limits (WELs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Long-term exposure level (8hrTWA): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Short-term exposure level (15 mins): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Respirable Dust – 15min STEL 5mg/m3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Detail Potential Health Risks: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Skin contact: There may be mild irritation at the site of contact. Eye contact: There may be irritation and redness. Ingestion: There may be irritation of the throat. Inhalation: There may be irritation of the throat with a feeling of tightness in the chest. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Control & Recovery Measures Actioned: (for example extraction, ventilation, training, supervision). Include special measures for vulnerable groups, such as disabled people and pregnant workers. Take account of those substances that are produced from activities undertaken by another employer’s employees. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ventilation: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Accidental Release & Spillage Plan : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No Specific measures. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Transfer to a closable, labelled salvage container for disposal by an appropriate method. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Special Transport: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Signage, provision of information and instruction : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Breifing this assessment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fire Precautions/Extinguisher Type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Environmental Considerations : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Suitable extinguishing media for the surrounding fire should be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Do not discharge into drains or rivers. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Storage Required: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Disposal Issues: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Store in a cool, well ventilated area. Keep container tightly closed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Disposal operations: Transfer to a suitable container and arrange for collection by specialised disposal company. Disposal of packaging: Dispose of as normal industrial waste. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Is health surveillance or monitoring required for foreseeable exposure potential? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Routine health screening |  |  |  |  | No |  |  |  |  | Personal air monitoring |  |  |  |  |  |  | No |  |  |  |  |  |  |  | Background air monitoring |  |  |  |  |  | No |  |  |  |
| Personal Protective Equipment (state type and standard) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item | Tick |  | Requirement |  |  |  |  |  |  |  |  |  |  |  |  |  | Item |  |  | Tick |  |  |  | Requirement |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Wear goggles to EN 166 is advised. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Use gloves conforming with EN 388 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Cover exposed skin. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| First Aid Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Skin contact: Wash immediately with plenty of soap and water. Eye contact: Bathe the eye with running water for 15 minutes. Ingestion: Wash out mouth with water. Inhalation: Consult a doctor. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Disposal Instructions for substance and empty containers: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous Waste |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | General Waste |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Return to Depot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Return to Supplier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other/Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Is exposure adequately controlled? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |  |  |
| Risk Rating Following Above Controls: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | High |  |  |  |  |  |  | Med |  |  |  |  | Low |  |  |  |