# Extension to form 8 new 2-bedroom flats to the rear of:

# 83 – 95 Wendover Road, M23 9EG

# External Finishes Specification

1. External Walls

1.1 Brickwork (Below DPC and to Floor Level)

- Damp Proof Course Zone:

  - Two courses of red engineering bricks to be installed up to DPC level.

  - Bricks to be Class A or B engineering grade, providing high durability and low water absorption.

- Brickwork to Floor Level:

  - Above the engineering brick courses, walls to be constructed in Scotch Common bricks up to finished floor level.

  - Brickwork to match existing property tones where applicable.

1.2 Rendered Elevations

- All elevations above floor level to receive a white K-Rend finish.

- Render system to be through-coloured, providing a durable, low-maintenance surface.

- Finish to be smooth or scraped texture (as agreed with client/LA), consistent across all elevations.

## 2. Windows and Doors

- All new windows and external doors to be white uPVC.

- Frames to be energy-efficient, double-glazed units meeting current Building Regulations, installed as per drawing 0824/01/54 (Condition 4)

## 3. Roofline Components

### 3.1 Fascias and Soffits

- New fascias and soffits to be white uPVC.

### 3.2 Rainwater Goods

- All gutters, downpipes, and associated fittings to be black uPVC.

- System to be half-round or square profile, installed to manufacturer’s recommendations.

## 4. General Notes

- All materials to be installed in accordance with manufacturer specifications and relevant British Standards.

- Any variations to the above specification must be agreed with the client and, where required, the Local Planning Authority.

- Samples of bricks, render, and uPVC components may be requested for approval prior to installation.