144229/FO/2025 Newton Heath Working Mens Club, 67 Old Church Street, Manchester, M40 2JN

CONTEXT

The application seeks consent for the erection of a new boundary fence to enclose the rear seating area, together with alterations to the car parking layout, landscaping, and provision of a bin and cycle space.

HIGHWAYS SUMMARY:

Highways are satisfied that the proposals contained within this application will not have an unacceptable impact on the local highways.

HIGHWAYS RESPONSE:

The site is located adjacent to the adopted highways Great Newton Street and Old Church Street.  Both streets have ‘no wating at any time’ kerbside restrictions and both are subject to a 20 mph speed limit.

TRIP GENERATION AND JUNCTION CAPACITY:

Trip assessment not appropriate for this application.

SITE ACCESSIBILITY:

The site is well accessed by sustainable modes with five bus routes available within a couple of minutes’ walk and a tram stop within a 10 minute walk.

OFF-SITE HIGHWAYS WORKS:

No off-site highway works are proposed.

PARKING AND ACCESS:

It is stated that there are seven existing parking spaces and these will be retained. However, viewed with Google Maps, the car park would be expected to be able to  accommodate a minimum of 10 cars.  Whilst the parking details contained within the application conflicts with what can be seen on site, the difference is considered minimal when taking an area wide view.

EV CHARGING

There are no additions to the car parking space numbers.  It is advisory only, that

the provision of one Electrical Vehicle charging point (minimum 7kW) is made available.

CYCLE PARKING:

Secure cycle parking provision will be made for three cycles within the curtilage of the site.  As seen on drawing 111_A1.01 rev P2.

BOUNDARY TREATMENTS:

The proposed fence measurements have not been provided.  However, on an understanding that the fence will typically be 2 metres in height, that the location of the new fence line will follow the existing fence line and the length of the new fence is as shown on plan.  Then the fence will not affect users on the highway nor drivers exiting the car park onto Great Newton Street.  Therefore, Highways are satisfied with the installation of the fence.

REFUSE AND SERVICING:

Designated refuse storage on site has been identified. Highways consider this to be acceptable in order to prevent any obstruction to passing pedestrians on the adjacent footway.

CONSTRUCTION MANAGEMENT:

A Construction Management Plan is not required, as any potential works can be accommodated off the highway.