MAINTENANCE INSTRUCTIONS & CLEANING FOR GALVANISED STEEL

Product  :-  Steel handrails / steel / gates /rails/staircase

Material :- Mild Steel

Coating  :-  Galvanised Finish (Hot Dip) & powdercoated

All Material is galvanised BSEN1461.

- To maintain the quality of the items, avoid any scratching of the surfaces, this will ensure that no corrosion occurs to any of the steel.

- To clean the steel  use of water and simple soap or disinfectant without scrubbing or scratching of the surfaces will maintain the coating.

- Should any scratches or breaks in the coating occur, preventing corrosion can be ensured by use of either a zinc primer or galvanising touch up paint or recoating with powder to match where required.