| DATE: 14/04/2026 | AUTHOR: Glyn Carmody |
| --- | --- |

| REF: 342 Slade Lane, Manchester M19 2BW | ACTIVITY: Complete demolition of former car workshop & outbuildings at 342 Slade Lane, Manchester M19 2BW |
| --- | --- |

ORGANISATION/COMPANY IN CONTROL OF OPERATION # Skip Hire All Waste Ltd # 24 Avondale Rd # Edgeley # Stockport # Cheshire # SK3 9NX

NAMED INDIVIDUAL RESPONSIBLE FOR ACTIVITY AND ITS SAFETY Glyn Carmody

LOCATION OF ACTIVITY INCLUDING ACCESS AND RESTRICTIONS 342 Slade Lane, Manchester M19 2BW Access is currently unrestricted to and from the site. Access will be from Slade Lane.

ARRANGEMENTS FOR CHANGING/DEVIATING FROM METHOD STATEMENT Consultation with client and construction safety consultant

GENERAL DESCRIPTION OF ACTIVITY Strip off roof sheets by hand, load into road transport and remove from site. - Men using hand tools will strip off roof sheets and under board, with the aid of a tower scaffolding, ladders etc. Load into skips provided and remove from site. - Men using hand tools will carefully cut out roof trussers, load into skips provided and remove from site. - Men using hand tools, with the aid of 20 tonne excavator will take down the wall adjoining the dwelling, to the height to be agreed with client, load into skips provided and remove from site. - Men with the aid of 20 tonne excavator, with selecta grab & bucket, will take down the remainder of building, process waste, load skips provided and remove from site. - Men using excavator will breakout concrete slab & footings, crush with crusher bucket to 6F2, leave on site for the re-use by client. - Upon completion site will be left in a clean and tidy manner.

OPERATIVES & TRAINING REQUIREMENTS Competent, trained, and experienced operatives (see training records)

GENERAL WORKING ENVIRONMENT CONSIDERATIONS Supervisor to control all operations Weather conditions Asbestos Containing Materials (ACM’s) Welfare to be provide by contractor. Light

PROTECTION OF OTHERS WHO MAY BE AFFECTED BY THE WORKS Complete exclusion zone to be set up and monitored in accordance with BS 6187:2000 Clause 13 – Safe Working Spaces & Exclusion Zones Conspicuously sign the existing building and external perimeter exclusion zone fencing indicating DANGER STRUCTURE and DANGER – DEMOLITION IN PROGRESS Vigilantly supervise

EMERGENCY PROCEDURES INCLUDING LOCATION OF EMERGENCY EQUIPMENT First Aider(s) on site Fire Fighting Equipment

PLANT & EQUIPMENT 20 Tonne Excavator with Selecta Grab, Crusher Bucket & Bucket Tower scaffold Steps Welfare provided by contractor. 4-leg and 2-leg chains with hooks and D shackles Hand tools and power tools

MATERIALS n/a

STORAGE n/a

SAFETY CRITICAL STAGES Disconnected and isolation of services Protection of utility meters and/or equipment identified on site and external to the property Restricted access Asbestos Demolition Survey Overhead wires/cables Dust and asbestos fibre suppression and containment Ground conditions Weather conditions Integrity of exclusion zones Access and egress Traffic management and vehicle supervision/banksman Plant operation checks Records of Thorough Examination PUWER 1998 – Records of Inspection Training provenance

SATISFACTORY COMPLETION TO SPECIFICATION/IMPOSED CONDITIONS/LEGAL COMPLIANCE To be signed off by client.

ADDITIONAL INFORMATION

PRINT, SIGN & DATE

Start date of work TBA.

Completion date within 3 weeks of commencement.