| Proprietary Treatment Systems |  |  |
| --- | --- | --- |
| Maintenance Schedule | Required Action | Typical Frequency |
| Routine Maintenance | Remove litter and debris and inspect for sediment, oil and grease accumulation | Six monthly |
|  | Change the filter media | As recommended by manufacturer |
|  | Remove sediment, oil, grease and floatables | As necessary – indicated by system inspections or immediately following significant spill |
| Remedial Actions | Replace malfunctioning parts or structures | As required |
| Monitoring | Inspect for evidence of poor operation | Six monthly |
|  | Inspect filter media and establish appropriate replacement frequencies | Six monthly |
|  | Inspect sediment accumulation rates and establish appropriate removal frequencies | Monthly during first half years of operation, then every six months |

| Bioretention Systems |  |  |
| --- | --- | --- |
| Maintenance Schedule | Required Action | Typical Frequency |
| Regular Inspections | Inspect infiltration surfaces for silting and ponding, record de-watering time of the facility and assess standing water levels in underdrain (if appropriate) to determine if maintenance is necessary | Quarterly |
|  | Check operation of underdrains by inspection of flows after rain | Annually |
|  | Assess plants for disease infection, poor growth, invasive species etc and replace as necessary | Quarterly |
|  | Inspect inlets and outlets for blockage | Quarterly |
| Regular Maintenance | Remove litter and surface debris and weeds | Quarterly (or more frequently for tidiness or aesthetic reasons) |
|  | Replace any plants, to maintain planting density | As required |
|  | Remove sediment, litter and debris build-up from around inlets or from forebays | Quarterly to biannually |
| Occasional Maintenance | Infill any holes or scour in the filter medium, improve erosion protection if required | As required |
|  | Repair minor accumulation of silt by raking away surface mulch, scarifying surface of medium and replacing mulch | As required |
| Remedial Actions | Remove and replace filter medium and vegetation above | As required but likely to be > 20 years |